Daylighting (Daylit spaces)

Visiting NRL was a great experience to see daylighting practices in action and the successes and failures of different designs. The first building we saw, the research support center, had a lot of daylighting successes as it is one of the newest buildings completed on campus. Three long, thin, east-west wings ensure maximum daylight coverage throughout the offices and allow the engineers to develop daylighting solutions to control daylight rather than struggling to bring more in. While one of the wings supports their supercomputer and doesn’t need windows, the other two spaces house open office spaces. These spaces use a reflective light shelf to redirect up and deeper into the space. This is controlled by electrochromic daylighting glass, whose goal is to limit very bright daylighting from affecting occupants in the space, but unfortunately it is not as effective as needed, so blinds were placed over them and are operated twice a year. The vision glass below provides views controlled by large fins on the sides and top of the windows and have no controllable daylighting sections.

The Icheon International Airport in South Korea was a wonderful example of daylighting in a massive public space. Large opaque (I think) skylights spanned across the two main connection points of the terminal, creating a fresh ambient light that was never overpowering. The fading out of open windows allowed the roof to seamlessly transition back into the rest of the airport rather than cutting the light of harshly. In other spaces, large glass windows led to outdoor areas, with the top half of the windows being controlled automatically with shades and photocells. In conjunction with the lighting, the large number of tall windows, overhead ambient daylight, and access to direct sunlight in specific locations, the space felt very comfortable and was a lovely place for our 8-hour layover. The maintenance on these windows and blind systems must be intense, as dirt, dust, and humidity would severely limit the daylight from the skylights, so there must be a robust system in place with either carefully selected materials or actively cleaning.